MAXTERM

New Micros, Inc.

09/28/95

INTRODUCTION

MaxTerm is a simple DOS-based communications package designed for

program development on serial port based embedded controllers. It can

run under stand-alone DOS or in a DOS session under Windows.

MaxTerm provides:

1. Support for COM1 through COM4.

2. Baud rates from 300 through 38400.

3. Control over RTS and DTR lines.

4. Capture files, which record all terminal activity to disk.

5. 32K scroll-back buffer, editable and savable as a file.

6. On-line Interactive Programmer's Editor (OPIE).

7. File downloader,

a. can trap errors in MaxForth

b. has a user-definable line pace character for compatability

with the Buffalo monitor and Basic, etc.

8. Programmable function keys.

9. Received character monitor, which displays all data in HEX.

MAXTERM.CFG

MaxTerm generates a configuration file called MAXTERM.CFG.  This file

contains all the operational parameters of the MaxTerm environment.  It

is updated each time MaxTerm is exited and loaded each time MaxTerm is

started.  The .CFG file may be edited by any standard ASCII file editor.

Note that the .CFG file cannot be successfully edited with OPIE since

any any changes made will be over-written when MaxTerm exits.

There are two parameters in the .CFG file that are not controllable from

within the MaxTerm environment:  12/24 hour clock display and the

tabstop size.  To change these, edit the .CFG file.

Tabstop setting:  The tabstop may be set to any integer between 1 and 63.

3 EDITOR TO TABSTOP FORTH

Clockmode setting:  The clockmode may be either 0 or 1.  The value is

actually interpreted as "zero or non-zero" with any non-zero value

accepted as "1".

0 TO CLOCKMODE ( 0=24 1=12 HOUR FORMAT)

QUICK START TUTORIAL

In DOS, simply copy the MAXTERM.EXE file to a directory in your path.

Then type

C:\> MAXTERM <enter>

MaxTerm will start, displaying a startup screen (similar to):

�������������������Active COM port

�     �������������Baud Rate

�     � �����������DTR on

�     � �����������RTS on

�     � �� ��������Capture on

�     � �� ��������Sounds active

�     � �� ��������Redef capture

�     � �� ��������Pacing active

�     � �� ����  ��time

�     � �� ����  �

������������������������������������������������������ͻ

�         � MAXTERM v1.0 � COM1   300 dr LSKP� 15:18:57�

������������������������������������������������������Ķ

�                                                      �

�                                                      �

�                                                      �

�     �����������������������������������������ͻ      �

�     �    MAXTERM (C) 1995 NEW MICROS, INC.    �      �

�     �   Press ALT-H for help, ALT-X to exit   �      �

�     �����������������������������������������ͼ      �

�                                                      �

�                                                      �

�                                                      �

�                                                      �

������������������������������������������������������ͼ

From here, either press ALT-H to see the MaxTerm help screen, ALT-X to

quit back to DOS, or any other key to enter MaxTerm.

Next, select your com port.

ALT-1 selects COM1

ALT-2 selects COM2

ALT-3 selects COM3

ALT-4 selects COM4

Then select your baud rate by pressing ALT-B until the baud rate value

on the status line is what you want it to be.  Most New Micros products

require 9600 baud.

That's all it takes!

In Windows, either start a DOS session and follow the DOS example, or

create a new program item using the MAXTERM.PIF file and double-click on

the newly created item.

CONTROLLING MAXTERM

MaxTerm is controlled with the PC's ALT-keys.  These functions are listed

in the Help screen (activated by ALT-H).  Note: in all filename prompts,

pressing the ESC key will cancel the operation.

������������������������������ MAXTERM Help ���������������������������ͻ

� alt-B Change baud rate               alt-M Character monitor mode     �

� alt-C Open (or close) capture file   alt-O Toggle sounds              �

� alt-D Download a file (all text)     alt-P Change line pace char      �

� alt-E Edit a file (Split screen)     alt-R Toggle RTS                 �

� alt-F Edit function keys             alt-S Unsplit the screen         �

� alt-H Help                           alt-T Toggle DTR                 �

� alt-I Program Information            alt-U Change colors              �

� alt-K Toggle redefinition catcher    alt-W Wipe the screen            �

� alt-L Open scrollback log            alt-X Exit                       �

� alt-1 (2 3 4) Select Com port        alt-Z Download a file (no fat)   �

� f1-f10 Programmable function keys    f12   Re-enter OPIE              �

�                                           ��������������������������� �

� Status line mode indicators:              � (C) 1995 New Micros, Inc. �

�   r = rts                                 � 1601 Chalk Hill Road      �

�   d = dtr                                 � Dallas, TX 75212          �

�   L = log file                            � info@newmicros.com        �

�   S = sounds                              � (214) 339-2204 voice      �

�   K = redefinition                        � (214) 339-1585 fax        �

�   P = line pacing active                  � (214) 339-2321 bbs        �

�����������������������������������������������������������������������ͼ

ALT-B   Changes the baud rate in power-of-two steps.  The valid baud

rates are 300, 600, 1200, 2400, 4800, 9600, 19200, and 38400.

ALT-C   Toggles the capture file state on or off.  A filename prompt

will appear each time the capture mode is turned on.  If the

file selected already exists, MaxTerm will append its capture

data to its end. If it doesn't exist, it will be created.

Capturing a large file in Windows should be run only in

full-screen mode.

ALT-D   Selects a file for download.  The file is sent to the board a

line at a time.  If line pacing (ALT-P) is selected, the

downloader will wait for its pace character before continuing to

the next line.  If no pacing is selected, all lines are sent

down as quickly as possible.  Downloads may be interrupted by

pressing the ESC key.

If you are downloading to MaxForth, any error will pause the

download and ask for "continue or abort".  A non-unique warning

will pause (if selected via ALT-K) and ask "continue, ignore, or

abort".  Continue means: go on loading, but pause on future

warnings.  Ignore means: ignore all other non-unique warnings.

See ALT-Z for an alternate "smart" download function.

See ALT-P for more discussion on downloading.

ALT-E   Invokes OPIE.  See OPIE FUNCTIONS.

ALT-F   Invokes the function key editor.  See FUNCTION KEYS.

ALT-H   Displays the help screen.

ALT-K   Toggles the pause on "not unique" warnings.

ALT-L   This function causes OPIE to be started with the scrollback

buffer in it.  You may edit this buffer normally and save it as

a file or reload it.

ALT-M   Toggles the character monitor mode on or off.  When it is on,

all data received by MaxTerm over the serial port will be

displayed as hex data, similar to a memory dump.

ALT-O   Toggles sounds on or off.

ALT-P   Brings up a selection box for choosing the download line pace

character.  If the selected character is zero ($00), pacing is

considered off.  Non-zero values are on, and represent the

character that the downloader will wait for at the end of each

line transmitted.  These two basic modes cover most situations.

With pacing off, characters are sent blindly to the remote. This

is suitable for downloading S19 records to the BUFFALO monitor.

With pacing on, the downloader is assuming that each character

transmitted will be echoed by the remote and checks for this

match. At the end of each line, a carriage return ($0D) is

transmitted, and the pace character is waited for.  Other

characters received during this wait are displayed and ignored,

with the exception of the question mark character ($3F) which is

interpreted as an error.

Basic typically requires a pace character of ">" ($3E).  Forth

requires a pace character of "line feed" ($0A).

The pace character is selected by pressing the plus "+" or minus

"-" keys on the keyboard until the desired value is displayed.

ALT-R   Toggles the state of RTS.

ALT-S   Cancels the OPIE split-screen mode, leaving the edited text visible

until normal scrolling pushes it off the screen.

ALT-T   Toggles the state of DTR.

ALT-U   Invoke the MaxTerm color editor.  This allows you to select the

color of each part of the MaxTerm environment.

ALT-V   Download the Windows clipboard.  The contents may be up to 32k bytes.

ALT-W   Wipe the screen.

ALT-X   Exits MaxTerm.  A configuration file (MAXTERM.CFG) containing

- the current color selections,

- comport,

- baud rate,

- function key definitions,

- pace character, and

- last filename

is written in the current directory.

ALT-Z   Like ALT-D except that ALT-Z trims the "fat" from the file before

downloading it.  "Fat" is:

- blank lines

- lines which begin with a comment characte, either "(" or "\"

- leading and trailing spaces on each line

This download function can be much faster than ALT-D, but is

perhaps not as predictable.

ALT-1   Select which COMM port to use. Also ALT-2 ALT-3 and ALT-4.

F-keys  F1 through F10 will transmit their definition (see FUNCTION

KEYS) when pressed.

F12     Re-enters OPIE in exactly the same position as was exited.

OPIE OVERVIEW

MaxTerm includes an integrated editor OPIE (on-line programmer's

interactive editor) for program development which makes it un-necessary

to exit MaxTerm just to make that _ONE_ change to your program.  OPIE

operates in "split-screen" mode, which means that a portion of the

terminal window is still visible while you are editing (although the

terminal will not display any serial data received).  When you exit

OPIE, the default mode leaves the text that you were editing on the

screen in an un-scrolling window.  You may cancel this split-screen mode

with the ALT-S function key in MaxTerm.

OPIE is a very simple editor, designed for quick editing jobs while

online developing code.  It can handle files as large as the available

memory, typically 300K or more.

Editing takes place in a 16 line by 80 column window at the top of the

screen with the MaxTerm status line above it and the OPIE status line

below it.  At any time, OPIE can be exited (with ESC) and re-entered

(with ALT-F12), allowing quick edit-download-test cycles.  An added

feature (F1) allows the current _VISIBLE_ edit text to be downloaded as

you exit the editor.  This means that portions of a file may be

downloaded for testing without having to download the entire program

every time.  The net result is similar to the way Forth historically

used BLOCKS for quick edit-compile-test cycles.

OPIE does not automatically do anything.  There are no backup files

created, no timed autosave, not even save-on-exit unless you use the

ALT-X command.  It is a very terse, simple editor.

OPIE FUNCTIONS

�������������������������� OPIE Help ����������������������ͻ

�      F1 exit OPIE, send all VISIBLE text to remote        �

�     ESC exit OPIE, don't save file (also F12)             �

�   ALT-x exit OPIE, save file to disk                      �

�   ALT-y save file, don't exit                             �

�                                                           �

�    ^e up line        ^r up page        ^w up screen       �

�    ^x dn line        ^c dn page        ^z dn screen       �

�    ^s left           ^a left word                         �

�    ^d right          ^f right word     ^i tab right       �

�    ^g del char       ^t del word       ^y del line        �

�    ^v toggle insert  ^l search again   ^u undel line      �

�                                                           �

�    ^qf search                                             �

�    ^qr top of file   ^qc bot of file                      �

�    ^qs beg of line   ^qd end of line                      �

�                                                           �

�  Arrows, home, end, insert, del, pgup, pgdn as expected.  �

�                                                           �

�����������������������������������������������������������ͼ

FUNCTION KEYS

MaxTerm supports 10 user-definable function keys mapped to the F1

through F10 keys on the PC keyboard.  Function keys may be edited while

online and are saved when MaxTerm is exited.  The function key

definition window can be opened with the ALT-F key while in terminal

mode.

A function key definition may contain normal ascii text and control

characters.  The backslash character ("\") indicates that a control

character follows.  For instance, a control-a is entered with the string

("\A").  Also, arbitrary hex codes may be entered via a

backslash-dollarsign. For example the page-feed character, which is a

12, is represented as ("\$0C"). Two backslashes in a row ("\\")

indicates a single backslash in the string.

To edit the function keys, press ALT-F.  This will bring up a window

showing each of the function key's current definition.

������������������������� Edit Function Keys �����������������������ͻ

�  1                                                                 �

�  2                                                                 �

�  3                                                                 �

�  4                                                                 �

�  5                                                                 �

�  6                                                                 �

�  7                                                                 �

�  8 HEX FF B000 C!\M                                                �

�  9 HEX 00 B000 C!\M                                                �

� 10                                                                 �

�                                                                    �

�     Press a function key to edit its definition, escape to exit    �

�                                                                    �

��������������������������������������������������������������������ͼ

From this window, you would press one of the function keys (F1 thru F10)

to edit its definition.  While editing a function key, the following

keys are active:

RIGHT   move the cursor right.

LEFT    move the cursor left.

HOME    move the cursor to the left edge.

END     move the cursor to the right edge.

INS     toggles insert/overstrike mode.

DEL     delete the character under the cursor.

ESC     exit without saving changes.

ENTER   exit saving changes.

In the example above, the F8 key is mapped to "HEX FF B000 C!\M". When

F8 is pressed, this string will be sent to the serial port.  The "\M" at

its end signifies that the "enter" key, or a carriage return, is to be

sent also.  The net effect is that of having typed the entire string and

pressing the enter key.